Madagascar Integrated Landscape Assessment

Methodology supplement

Adrian Leigh Vogl , Evariste Rutebuka, Carlos Alberto Solis Rojas, Jorge Eduardo Leon Sarmiento, Urvashi Narain

April 2022

# Study Objectives

The objectives of the current study are to

- Create a spatially explicit, data driven assessment of land degradation in Madagascar over the last 30 years;

- Evaluate the impacts of land degradation on ecosystem services to key sectors including erosion control for agricultural productivity, water and sediment regulation for hydropower production, and carbon storage;

- Evaluate the potential for investments in landscape management that improve vegetation cover to effectively offset landscape degradation and improve ecosystem services values to key sectors; and

- Identify priority areas for investment in NBS based on their potential contribution to improving ecosystem services and landscape productivity.

The overall methodology comprised the following steps, which are detailed in the sections below:

- Using data on physical conditions (topography, soils) along with historical data on land use land cover, rainfall, deforestation, and remote sensing-based ecosystem indicators, perform a trends analysis to identify areas of where erosion and ecosystem health indicators are in decline, and where they show recovery.

- Combine information on land degradation and land use change with publicly available economic information to estimate the economic impact of land degradation, focusing on the agriculture, hydropower generation, and carbon emissions sectors.

- Apply spatially explicit ecosystem services models to evaluate the potential impact of investments in improved land management (ILM) on baseflow, storm runoff, and erosion.

- Apply the models developed in step 2 to estimate the potential savings to key sectors that could result from these investments.

# Trend analysis of land degradation

A composite land degradation index was developed based on pixel-level analysis of historical trends in four sub-indicators: vegetation health condition, carbon storage, soil retention, and baseflow contribution. These sub-indicators were selected based on FAO’s Global Land Degradation Information System (GLADIS) method, which emphasizes an assessment of the status and trends of ecosystem goods and services, including the impacts that changes have on local populations 1,2. The selected indicators rely on remote sensing data and are known to provide meaningful proxies of land degradation in terms of severity, temporal dynamics, and areal extent 3.

Selected sub-indicators for the composite land degradation index 4:

- Vegetation health condition: The Normalized Difference Vegetation Index derived from Landsat data (NDVI; 30m resolution) was used as a proxy for vegetation health. NDVI provides valuable information about temporal and spatial changes in vegetation distribution, productivity, and dynamics, allowing monitoring of habitat degradation and fragmentation, or assessment of the ecological effects of climatic disasters such as drought or fire 5.

- Carbon storage: MODIS-derived Net primary productivity (NPP; 500m resolution) was used as a proxy for carbon storage. NPP data indicates inter-annual variations in net biomass (carbon) gain by vegetation over year-by-year. Changes in this indicator also point to how the landscape has changed in terms of the availability of food, biofuel, and fodder over time6, and has been used as a measure of agricultural sustainability 7.

- Soil retention services: The InVEST Sediment Delivery Ratio model v3.9 8 was used to model soil retention for this indicator (90m resolution). The model output represents the capacity of a landscape to retain sediment by using information on geomorphology, climate, vegetative cover, and management practices such as anti-erosive techniques available in the landscape.

- Water availability via baseflow: Baseflow refers to water slowly released as surface water from local recharge of soil moisture and aquifers 9. Baseflow was modelled using the InVEST Seasonal Water Yield Model v3.9 30. Baseflow is mainly driven by precipitation and evapotranspiration and vegetation cover management present in the landscape 10.

The final Land Degradation composite index was calculated by combining the four sub-indicators as described in Figure 1. The land degradation index was supplemented with an analysis of deforestation and changes in precipitation over the same time period, to explore their linkages to land degradation. We also report on human population change, livestock density, and deforestation/ land degradation within 500-meter buffer zone of riverbanks.

Figure 1: Madagascar Land Degradation Index based on historical trends from 1992 to 2020.

## Data processing

The vegetation health indicator utilized the Normalized Difference Vegetation Index (NDVI) derived from Landsat imagery at 30-meter resolution, available from 1992-2020. Net Primary Productivity (NPP) at 500m resolution was obtained from MOD17A3HGF V611, for the years 2000-2018. Five-year moving averages of NDVI were calculated to account for the fact that inter- and intra-annual variability of external factors can affect the values of these parameters year-to-year (for example,  fallow periods that range from 3 to 5 years in Madagascar 12, slash-and-burn agriculture (tavy), seasonality and plant phenology, etc. 13,14). For example, the NDVI values estimated for 1992 were the average NDVI based on all imagery recorded 1990-1994, then NDVI for 1993 were the averages from imagery recorded from 1991 to 1995. The same moving window averaging was applied to NPP data. Historical series of NDVI were corrected to account for differences in sensor specifications between Landsat 5, Landsat 7, Landsat 8. The NDVI transformation equation for intercalibration of vegetation indices from different sensor systems was applied, based on studies of Steven et al. 15 for Landsat 5 to 7, then Roy et al.16 for Landsat 7 to 8.

The analysis of soil retention and baseflow utilized the InVEST Sediment Delivery Ratio (SDR) model and Seasonal Water Yield (SWY) models, v3.9. InVEST is a widely used tool for ecosystem services modelling, and has been tested in high or poor data environments with promising results 17,18. The spatially explicit SDR model estimates for each pixel the average amount of erosion per year, then integrates information on the landscape context (land cover and land use upslope and downslope of the pixel) to estimate the amount of sediment thereafter retained on the landscape washed away in streams. The model is based on an implementation of the Revised Universal Soil Loss Equation (RUSLE1)46 for the calculation of annual soil loss, and a sediment delivery function driven by the hydrological connectivity of each pixel within the landscape. Once sediment reaches the stream, the model assumes that it ends up at the catchment outlet, thus no in-stream sediment deposition or remobilization processes are modelled 19.

The seasonal water yield (SWY) model allows users to rank specific parcels (or pixels) of land for conservation or development based on their relative contributions to specific components of the hydrological cycle. The model partitions precipitation into either quick flow or base flow (runoff versus groundwater recharge) by calculating a water balance for each individual pixel of the watershed of interest. The information required by the SWY model is easily obtained globally from publicly available data sources and includes monthly precipitation, topography, evapotranspiration, land-use, soil type, and land-cover data 20.

The required input data for the SDR and SWY models include topography (digital elevation model, or DEM), soil characteristics (erodibility and hydrologic soil group), monthly mean rainfall, monthly mean reference evapotranspiration, and various physically-based parameters that reflect how vegetation cover and condition affect the impact of rainfall on erosion and the partitioning of rainfall inputs into runoff, infiltration, and baseflow. See Section 5 for a full listing of data requirements and sources.

A key input is the land use land cover (LULC) map. Because an official national land cover map does not exist that satisfies the historical criteria for the study, the European Spatial Agency (ESA) Climate Change Initiative (CCI) land cover map at 300m resolution was used. This map is available on yearly basis since 1992 21,22. The Copernicus Global Land Service: Land Cover (CGLOPS; 100m resolution) and the recently released 10m resolution land cover map by ESA 23 were considered, but were rejected because they were not available over the historical time period, despite their high spatial resolution. The 38 native classes in the ESA CCI LULC maps were reclassified to align with the 11 classes of ESA 23 to allow compatibility of future LULC developed by Madagascar Land Use Planning for Enhanced Resilience of Landscapes (LAUREL) project completed in July 2020 24 and 10-m resolution global map class.

Precipitation and reference evapotranspiration were taken from the Climate Hazards Group InfraRed Precipitation with Station data (CHIRPS) 25,26. Rainfall erosivity was taken from Panagos et al. (2017), who estimated global erosivity based on extensive field-based data from 3,625 stations observed for 16.8 years 27. Soil erodibility (K) values were based on Africa soil typology from Hengl et al. (2021) at 30m resolution, which provides fractions of silt, fine sand, clay, organic content, and bulk density parameters  28. K values were calculated according to the EPIC model methodology (Equation 2) 29,30 and converted into international units by multiplying by 0.1317 31

(Equation 2)

where

San : sand content (%)

Sil : silt content (%)

Cly : clay content (%)

C : soil organic carbon content (%)

Other parameter inputs to the models were guided by local studies and regional studies whenever available (see model parameter values in Section 6). The SDR and SWY models were run yearly from 1992 until 2020, and the resulting output rasters for soil retention (sed_retention, tons/ha/yr) and baseflow (B, mm/yr) were used in the subsequent trend analysis.

Deforestation between 2000 and 2020 was analyzed based on yearly forest cover from 2000 to 2018 developed by The United States Forest Service (USFS) in partnership with Madagascar Bureau National des Changements Climatiques, et de la REDD+(BNCCREDD+). The original data classified forest into four major classes: Humid forest, dry forest, mangrove, and thorny forest, and those classes were combined to estimate forest coverage. To explore deforestation, and to control for inter-annual forest dynamics and potential discrepancies in forest classification year-to-year, we subtracted the forest cover of 2018 from the area that has ever been classified as forest between 2000 and 2017 to obtain the total area deforested.

## Trend analysis: Slope, magnitude, and change significance

For each of the four indicators plus precipitation, a trend analysis using pixel-based linear regression was performed using the processed time series (data and/or model results). The resulting slope of the regression indicates the annual increment of the indicator (either positive or negative), while the regression intercept value indicates the magnitude of the specified indicator.

The slope, which represents the time variation characteristics of variables, was determined by using ordinary least squares regression pixel-by-pixel using Equation 3 32,33. The slope is expressed as follows:

(Equation 3)

where

n : number of years studied

i : year

θ : regression slope

The indicator's magnitude was calculated as the trendline intercept as shown in Equation 4:

A=y-θ x̄                                           				     (Equation 4)

where

A is the intercept

ȳ represents the mean value of NDVI ,NPP, precipitation, soil retention or baseflow volume

θ is the trendline slope

Estimated trends that were significant at the 95% confidence level (based on T-values) were used to classify the pixels into degrading or improving, and those that were not significant were classified as no change or no trend. The correlation coefficient was used to calculate the T value (T-test) 3435, and the T-distribution table was used to reclassify the T-value at 95% confidence interval in relation to the degree of freedom of each indicator. The correlation was calculated by pixel-by-pixel correlation analysis, using equation 5  32,33.

(Equation 5)

where

n : number of years studied

x̄ : mean value of the independent variable x

Ȳ : mean value of NDVI, NPP, precipitation, soil retention or baseflow volume

rxy : correlation coefficient between the variables x and y

The T-value was calculated using equation 6.

T=Rn-21-R2                                                                         	    	                (Equation 6)

where

T : t-value

R : correlation

n : number of years studied

## Land degradation composite index

The land degradation composite index was calculated as a weighted index developed through three stages. The first stage excluded any pixels where all four sub-indicators showed no trend over the study period. The second stage considered the annual increment (e.g., slope of the regression) for each sub-indicator. The increment was classified into one of six classes and pixels in each class were assigned the corresponding score: highly degrading (-4), moderately degrading (-3), slightly degrading (-2), slightly improving (+2), moderately improving (+3), and highly improving (+4). Pixel-level scores for the four sub-indicators were summed. For example, if a pixel showed moderate degradation in NPP (-3) but showed a high improvement in baseflow (+4), this led to a positive score (+1), implying an overall positive trend between the two sub-indicators. The third stage accounted for the magnitude (or the potentiality) of the services, indicating the area where a small change could have a high impact. For example, the soil retention service has a high magnitude in steeply sloping regions where there is also high precipitation. Thus, any proportional change on that high magnitude pixel would lead to a high impact relative to a low magnitude pixel, even though the proportional change might be the same. The magnitude value was represented by the intercept of the regression trendline. The magnitude was similarly classified into four scores based on quartiles, and then all four sub-indicator magnitude scores were combined in an additive approach.

The final land degradation composite index is the product of the final increment (slope) and magnitude scores, and was reclassified into six classes (three degrading and three improving; refer to Figure 3).

# Estimating economic impacts of land degradation

Information on net primary productivity (carbon storage) and erosion generated by the above analysis was processed to estimate the economic impact of land degradation for Madagascar. Key sectors were analyzed with multiple economic valuation methods, inculding agriculture, hydropower generation and carbon emissions; at the National level and for the 29-year period between 1992 and 2020.

## 2.1 Carbon

### 2.1.1 Carbon stock losses

Land use change and land degradation result in the release of carbon stock due to deforestation and a decrease in the capacity of all ecosystems to absorb carbon from the atmosphere through photosynthesis.

The release of carbon stocks from diminishing forest covers has an opportunity cost for society, as it can no longer access funds from carbon credits in the cleared areas. Carbon credits are traded in different markets around the world, with USD $45 billion raised in carbon pricing revenues globally in 2019 (World Bank, 2020). Madagascar is not a stranger to this revenue source, for example with the Atiala Atsinanana Emission Reduction Program, Madagascar, that sets a price of USD $5 per ton of CO2e as reference (FCPP,2018). This potential market price and the carbon content of Madagascar’s forests can be used to estimate the potential income loss by Madagascar due to land use change.

We used Vieilledent et al. (2018) data on forest loss from 1992-2010, and data reported by Forestwatch (2021) for 2011-2020. The humid forest category was selected because most REDD+ projects are located in this type of ecosystem. Carbon stock release was estimated from two diferent sources that were used as two different scenarios for the valuation. The final value reported was the average of the two scenarios.  Both national level estimates as well as specific losses in Protected Areas were calculated, due to the importance of these critical zones. The value of potential carbon credits lost was calculated based on the following formula:

C = P * (A * S)						(Equation 7)

where

C: Opportunity cost of lost carbon credits

P: Market price of CO2, measured in $ per ton of CO2

A: Total area of deforestation, measured in ha

S: CO2 content of forest, measured in tons of CO2 per ha

2.1.1 Carbon absorption losses

The decreasing capacity of ecosystems to absorb carbon implies that significant amounts of this greenhouse gas are left in the atmosphere instead of being incorporated and transformed to biomass. The year to year changes in NPP were calculated in Section 1. The average amount of carbon not absorbed during the analysis period was 1.56 M tons of CO2 per year. Besides the potential carbon credits, an alternative approach to measure this impact is through the social cost of carbon. This cost of carbon incorporates the indirect negative effects of carbon emissions at a global level and into the future (Hockley & Razafindralambo, 2006). Nordhaus (2017) estimates that the social cost of carbon was US $38.1/ton CO2 in 2015. This reference value was updated to 2020 USD of the specific years of the study period based on Nordhaus’ (2007) recommendations, considering global growth rates as well as a 3% increase in value from year to year. Applying the social cost of carbon to the amount of CO2 not absorbed due to land degradation allows its valuation with the following formula:

C =   Nt x St					(Equation 8)

where

C: Global cost of carbon not absorbed

N: NPP decrease in period t, measured in tons of CO2 at the National level

S: Social cost of carbon price for period t, measured in 2020 US$ per ton of CO2

## 2.2 Hydropower

### 2.2.1 Hydropower generation

The impact of sedimentation and reducing water yields in the 3 main hydropower plants of Madagascar (Andakaleka, Antelomita and Mandraka, that provide nearly 90% of the generation in the Antananarivo Interconnected Network) was calculated based on the work by Adeogun et al.(2016). These authors explain the difference between the installed generation capacity and the actual generation based on multiple factors and determine that suboptimal hydrological provision accounts for 19% of generation losses.

Weighted matrix for factors related to loss in hydropower production

| Task | Mechanical | Hydrological | Electrical | Civil | Maintenance | Sediments | Others |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Ranking | 1 | 2 | 3 | 4 | 5 | 6 | 7 |
| Weighting | 60 | 35 | 30 | 15 | 20 | 10 | 10 |
| Normalizing (%) | 33 | 19 | 17 | 8 | 11 | 6 | 6 |

Source: Adeogun, et al 2016

Based on this coefficient, the historical generation of electricity and the tariffs for electricity, it was possible to estimate how much less reliable water is costing the hydropower sector. The following formulas detail the estimation process.

HPL = ICG-HPGt 						(Equation 9)

C = t=1nHPLt ⋅WMD ⋅ Pt			(Equation 10)

where

HPL: hydropower production loss, measured in kWh

ICG: Installed capacity hydropower generation, measured in kWh

HPG: Hydropower generated in period t

C: Cost of hydrological hydropower production loss,  measured in 2020 USD

WMD : Hydrological factor in hydropower loss

p: Selling price of electricity for t period, measured in 2020 USD per kWh

Two scenarios were calculated to estimate the generation losses for each hydropower plant. For Andekaleka, the first one used the difference between the installed capacity at a 95% efficiency rate and the actual generation, and the second scenario used the difference between the maximum historic generation and actual generation. For the other 2 dams, scenario 1 used the difference between capacity at a 90% efficiency rate and actual generation, and a second scenario that used the difference with the maximum historical generation. The final reported values are the average of the two scenarios. Historical electricity tariffs for the period between 2001 and 2020 were used, considering the distribution of clients between the different zones and the low, medium and high tension differences, each with their respective price tiers.

### 2.2.2 Potential dredging costs of reservoirs

The impact of land degradation on hydropower plants can also be calculted based on the cost of dredging the accumulated sediments. This methodology is often used for the estimation of land management costs of land degradation, specifically the cost of loss in hydropower production (see for example Udayakumara & Gunawardena, 2017).

The estimation method is described in the following formula :

C = S * D						(Equation 11)

where

C: Cost of dredging reservoir, measured in 2020 USD

S: Total sediment trapped in reservoirs, measured in tons

D: Cost of dredging, measured in 2020 USD per ton

For the sediment trapped, we used the sediment export result from the SDR model, summed for each dam's contributing watershed area. Total sediment arriving at each dam was adjusted to account for trapping efficiency of the reservoirs, following the approach of Brune (1953) which relates trapping efficiency as a function of reservoir storage capacity and contributing watershed area. Incoming sediment values were adjusted according to the estimated trapping efficiency for each dam as given in the table below, to calculate the actual amount of sediments trapped in each reservoir.

| Dam | Sediment trapping efficicency % |
| --- | --- |
| Ambilivily | 0.8725 |
| Amboromalandy | 0.5717 |
| Andekaleka | 0.8805 |
| Antanifotsy | 0.8426 |
| Antelomita * | 0.8161 |
| Mandraka | 0.8161 |
| Mantasao | 0.8977 |
| Sahamoloto | 0.7528 |
| Tsiazompaniry | 0.8947 |

*Two dams located upstream of Antelomita (Tsiazompaniry and Mantasao) also act to trap some sediments arriving from teh upper watersheds. Therefore, sediment inflows to Antelomita were further adjusted to account for trapping efficiencies of upstream dams, as follows: Antelomita =Sediment trapping efficiency Antelomita x [sediment export Antelomita - (sediment to export to Tsiazompaniry x sediment  trapping efficiency Tsiazompaniry) - (sediment export to Mantsoa x sediment  trapping efficiency Mantasao)]

To determine the price of dredging in Madagascar, an extensive literature review was performed. The review showed that prices of dredging range from US $3/ton to US $9.4/ton. Annandale et al. (2016) suggests US $3/ton as a price reference, and in the WEDA report (2021) the range for dredging price over the last decade has been US $3.5 to US $5.8/m3. Adeogun et al. (2015) uses a price of US $9.4/m3 as the official price according to the National Inland Waterways Authority (NIWA). Grimaldi et al. (2013) uses a price of US $4.5/ton for Burkina Faso, a country with a similar economy to Madagascar.

Two scenarios were calculated to account for uncertainty in estimating the dredging cost, since specific information for Madagascar was not available: scenario 1 assumes a unit cost of dredging of US $4.5/ton, which is the average cost from the literature review; and scenario 2 assumes a unit cost of dredging of US $9.4/ton (maximum price from literature review). Final reported values are the average of these two scenarios.

## 2.3 Agriculture

### 2.3.1 Productivity losses

One of the sectors most affected by land degradation is agriculture, due to the close relationship between soil health and productivity. A number of studies in Madagascar and other countries have proven that erosion, lower water availability and other land degradation indicators have a negative effect on yields. Some relevant papers that shed light on this issue are presented in the table below.

| Citation | Geographic focus | Main results | Methods |
| --- | --- | --- | --- |
| Carret, J; Loyer, D. (2003). Madagascar protected area network sustainable financing. Economic analysis Perspective. | Madagascar | 10% loss of productivity in rice plots, due to silted irrigation channels and suboptimal water availability | Field data analysis in Maroantsetra and Alaotra. |
| Randrianarisoa, J. & Minten, B. 2001. Agricultural Production, Agricultural Land and Rural Poverty in Madagascar | Madagascar | Cyclones on previous year reduce Agricultural production value on 7% (National Level estimate). | National Survey, statistical analysis, and market prices |
| Portela, R., Nunes, P. A. L. D., Onofri, L., Villa, F., Shepard, A., & Lange, G. M. (2012). Assessing an d valuing ecosystem services in the ankeniheny-zahamena corridor (caz), madagascar. A Demonstration Case Study for the Wealth Accounting and the Valuation o f Ecosystem Services (Waves) Global Partnership. | Madagascar | 1% increase in the use of water (rainfall or irrigation) leads to a 0.91% increase in the production of rice and a 0.83% increase in the production of manioc | Production function |
| Sartori, M., Philippidis, G., Ferrari, E., Borrelli, P., Lugato, E., Montanarella, L., & Panagos, P. (2019). A linkage between the biophysical and the economic: Assessing the global market impacts of soil erosion. Land use policy, 86, 299-312. | Global | Mean crop yields loss in areas of severe erosion of 8% in comparison to non-affected areas | Literature review |
| Pimentel, D., Harvey, C., Resosudarmo, P., Sinclair, K., Kurz, D., McNair, M., ... & Blair, R. (1995). Environmental and economic costs of soil erosion and conservation benefits. Science, 267(5201), 1117-1123. | Global | Severe soil erosion by water (rates of higher than 17 t ha−1 year−1) can cause a crop productivity loss of 8% annually. | Review article |
| Lal, R. (1995). Erosion‐crop productivity relationships for soils of Africa. Soil Science Society of America Journal, 59(3), 661-667. | Africa | Yield reductions due to severe erosion may range from 2% to 40%, with a mean of 8.2% for the continent. | A review of available data in African plots |

Pimentel et al. (1995), Sartori et al. (2019), and Lal, R. (1995) estimate that high erosion decreases agricultural productivity approximately 8% in comparison to areas with no erosion. Based on this coefficient, an estimate of the lost value in agricultural production due to land erosion was obtained by incorporating mean erosion at the regional level from the SDR model, prices of main crops, and regional data on agricultural production. The definition of what constitutes high erosion varies by country, with values ranging over a broad spectrum, depending on sources. For the Madagascar case, 2 scenarios were used to classify regions afflicted by erosion. The first scenario used the average value of 70 tons/ha as the threshold for eroded regions, while the second scenario considered regions with erosion of over 50 tons/ha as impacted. Final reported values were the average of the two scenarios. In the case of regions afflicted by erosion, the estimation of productivity losses was calculated based on the following formula.

C = nRit⋅L ⋅ Pit							(Equation 12)

where

C: cost of agricultural losses for erosion aflicted region, measured in 2020 USD

R: Total production of crop i in period t, measured in tons

L: Production loss coefficient

P: Price of crop i in period t, measured in 2020 USD per ton of produce

### 2.3.2 Potential dredging costs of irrigation dams

An additional impact of land degradation in the agricultural sector is the siltation in irrigation dams. Similarly, as the case of hydropower, removing the sediments accumulated in the reservoirs implies an investment proportional to the severity of the problem. The potential cost of dredging these reservoirs was calculated using the same method described above for hydropower dams. Similarly, 2 scenarios were calculated, using the average cost of dredging and maximum values obtained from literature review, as described in Section 2.2.2.

# Estimating benefits of integrated landscape management (ILM)

Spatially explicit ecosystem services models (as described in Section 1) were applied to estimate the potential improvement that could be achieved through implementing ILM in productive landscapes in Madagascar. The objective of the analysis was to identify the areas where the implementation of ILM practices can have the greatest effect in reducing soil loss, thereby preventing further losses in the productivity of croplands and grasslands, and in improving rainfall-runoff dynamics, thereby reducing peak flows (which exacerbate flood risk) and increasing base flows.

ILM potential was estimated using the same InVEST SDR and SWY models developed for the degradation trends analysis. The models were run for the baseline (2020) condition, and then again applying a scenario that reflects the potential implementation of ILM. The ILM scenario assumed broad investments in activities that improve the condition of vegetation and soil cover, such as agroforestry, silvopasture, revegetation, etc., and assumed that such activities would be focused in areas classified as croplands (cultivated and managed vegetation), grazing areas (grasslands vegetation), or in degraded areas classed as bare ground or shrubland. These land management practices are assumed to be implemented in all available locations in the country, reflecting what the landscape could look like if there was a strong commitment to implementing land management across the board. This approach allowed for highlighting the watersheds where full implementation brings the greatest potential benefits.

Model parameters reflecting the implementation of soil conservation activities and surface runoff management practices were used as input to the SDR and SWY models. ILM implementation was reflected in the SDR model by assuming a 15% improvement in the management factor for shrublands and bare areas, and a 40% improvement in croplands and grasslands. In the SWY model, changes were reflected as a 15% increase in water use by vegetation (reflecting more vegetation density and/or vigor), a 5% reduction in runoff curve number for shrublands and bare areas, and a 10% reduction in runoff curve number for croplands and grasslands (reflecting vegetation's ability to capture and infiltrate rainfall). See section 6 for model parameters. We assumed 2040 as the time frame for the ILM scenario, reflecting full implementation of improved practices.

The benefits of ILM are estimated as the change in the average annual erosion rate, sedimentation rate to major reservoirs, annual average baseflow, and annual storm surface runoff for each pixel between the baseline (2020) land cover and the scenario where ILM practices are implemented. The catchment areas for major dams were taken from the Global Reservoir and Dam Database (GRanD) v1.3 dataset (Lehner et al., 2011). Changes in erosion were summarized at the regional level to allow for valuation of benefits to crop productivity. Changes in annual sedimentation were summarized for each catchment area of the 3 major hydropower reservoirs (Andekaleka, Antelomita, and Mandraka) and the 6 major irrigation reservoirs (Ambilivily, Amboromalandy, Antanifotsy, Mantasao, Sahamoloto, and Tsiazompaniry).

The pixel level results for change in erosion, baseflow, and storm surface runoff from ILM implementation were combined into an index of ILM potential by first calculating the difference between the baseline and the ILM scenarios. For erosion and storm runoff, a greater reduction in the indicator value indicated a greater benefit of ILM. For baseflow, the sign of the change was reversed, to indicate that a greater increase in baseflow is an improvement. The mean of each indicator's difference was calculated at the district level. District level means were then classified based on quartiles (25th percentile, 50th percentile, 75th percentile, and 100th percentile) and assigned a value of 1 to 4. The final index of ILM potential is the sum of the component quantile scores.

Note that for this high-level screening analysis, we do not make any assumptions about the specific land management practices employed; rather, we assume that the most locally appropriate ILM practice will be selected and implemented, and the end result will be an enhancement of vegetation cover, water infiltration, and soil health. Details on the specific ILM practices that are most appropriate and effective for a given location are left for subsequent analyses, when we expect that locally specific data are available, coupled with stakeholder engagement on feasible and locally acceptable practices.

# Valuing ILM benefits

Restoring environmental integrity can also lead to economic gains. Based on the modeling described Section 3, it was possible to estimate the potential value of integrated landscape management for Madagascar. We focused on projecting potential gains for the agriculture and hydropower sectors over a 20 yea future period.

## 4.1 Hydropower and irrigation dams

The reduction of sediment exports in the watersheds of dams, achieved by the implementation of ILM practices would result in lower potential dredging costs. The valuation of the amount of total savings was calculated by first estimating the potential dredging costs without the implementation of ILM. For this we used the values calculated for dredging of sediments accumulated during 2020, multiplied it by two dredging costs (US $4.5 and US $9.4 per ton) and used the average of the two costs as the final value. We then assumed that this cost would remain constant until 2040.

Cost of dredging under ILM practices used the new estimates of sediment exports obtained from the ILM scenario, adjusted for the trapping efficiencies of reservoirs, and multiplied by the two possible dredging costs over the 20 year analysis period. The savings from implementing ILM practices for each hydropower and irrigation dam is the diference between the costs in the Business as Usual (BAU) projection and the average of the two ILM projections, as described in the formula below.

S =   CBAUt - CILMt								(Equation 13)

where

S: Savings from the implementation of ILM practices, measured in 2020 USD

CBAU: Cost of dredging a dam in period t, in the Business as Usual projection

CILM: Cost of dredging a dam in period t, in the ILM projection

## 4.2 Agriculture

The reduction in erosion from the implementation of ILM practices will also result in gains for the agricultural sector. The potential benefits that could be achieved were calculated by first estimating the potential productivity losses without the implementation of ILM. For this, we used the values calculated for agricultural losses in the different regions of Madagascar during 2020, from the results of section 2.3.1. We then assumed that these costs would remain constant until 2040.

To calculate potential agricultural losses under ILM practices, we used the new estimates of erosion obtained from the ILM implementation scenario. These estimates showed the final erosion to be expected by 2040 in the intervened regions. We then assumed that the decrease in erosion would be linear at a constant rate, reflecting a linear establishment and growth of vegetation over the 20 year period. This assumption allowed to project specific erosion values for each region and for each year between 2020-2040. The level of erosion in turn determines if a region would suffer losses from erosion degradation, just as in the case of the BAU projections. Similarly, two different levels of erosion define the threshold for losses, 70 tons/ha and 50 tons/ha, and the final value is the average of the two scenarios.

The gain in agricultural productivity from implementing ILM practices for a specific region in Madagascar is the difference between the productivity in the Business as Usual projection and the average of the two ILM projections, as described in the formula below, while total savings are the sum of all regions.

G =   AILMt - ABAUt 							(Equation 14)

where

G: Agricultural gains value, meassured in 2020 USD

AILM: Agricultural losses due to erosion in ILM projection in period t

ABAU: Agricultural losses due to erosion in BAU projection in period t

# Data sources used in the land degradation analysis

| Data type | Description | Date covering data | Resolution | File Format | Source | Source Link |  | Used For |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| General |  |  |  |  |  |  |  |  |  |  |
| DEM | Digital Elevation Model | 2000 | 90m | Raster | WWF Hydrosheds, Hydrological Cond. DEM | https://www.hydrosheds.org/ |  | SDR and SWY modelling |  |
| Land Cover & Land Use | LULC from ESA Climate Change Initiative (CCI) global land cover (CCI) | 1992-2020 | 300m | Raster | European Space Agency Climate Change Initiative (ESA CCI) | https://cds.climate.copernicus.eu/cdsapp#!/dataset/satellite-land-cover?tab=form |  | SDR, AWY and SWY modelling |  |
| Precipitation | Daily precipitation | 1992-2020 | 5Km | Raster | Global Precipitation Climatology Project (GPCP) Climate Data Record (CDR) Daily analysis | https://data.chc.ucsb.edu/products/CHIRPS-2.0/africa_monthly/tifs/ |  | SWY modelling |  |
| Administrative Units | Regions and District boundaries | 2018 | Admin level 1,2,3,4 | shapefile | The Humanitarian Data Exchange | https://data.humdata.org/dataset/madagascar-administrative-level-0-4-boundaries |  | SDR, AWY and SWY and modelling |  |
| Erosion (SDR) |  |  |  |  |  |  |  |  |  |  |
| Soil erodibility | Soil USLE_K erodibility coefficient derived from physical properties | 1950-2020 | 30m | Raster | Africa Soil Grids | https://www.nature.com/articles/s41598-021-85639-y |  | SDR modelling |  |
| Rainfall erosivity | Annual average rainfall erosivity | 2000-2010 | 1km | Raster | Rainfall Erosivity of the World | https://esdac.jrc.ec.europa.eu/content/global-rainfall-erosivity |  | SDR modelling |  |
| USLE_C | Crop cover factor, by land use/land cover type | 1990-2020 | 300m | Table | USLE_C crop/cover factor, obtained from regional studies and InVEST 3.9 model user guide recommendations | https://doi.org/10.1007/BF00889179 |  | SDR modelling |  |
| USLE_P | Management practice factor, by land use/land cover type | 1990-2020 | 300m | Table | Average USLE_P from two recent field based studied in Madagascar | - https://biblio.univ-antananarivo.mg/pdfs/rakotomamonjyTolojanaharyE_ESPA_MAST2_16.pdf - https://onlinelibrary.wiley.com/doi/10.1002/ldr.3016 |  | SDR modelling |  |
| Seasonal Water Yield (SWY) |  |  |  |  |  |  |  |  |  |  |
| Precipitation | Annual and monthly precipitation | Monthly precipitation since 1992 to 2020 | 5km | Monthly and annually | Climate Hazards Group InfraRed Precipitation with Station data (CHIRPS) | https://data.chc.ucsb.edu/products/CHIRPS-2.0/ |  | AWY and SWY modelling |  |
| Evapotranspiration | Reference evapotranspiration (et0) | Estimated from dekadal (10-day) et0 since 1992 to 2020 |  |  | Climate Hazards Group InfraRed Precipitation with Station data (CHIRPS) | https://data.chc.ucsb.edu/products/Hobbins_RefET/ |  | AWY and SWY modelling |  |
| Depth to Bedrock |  | 2021 | 30m | Raster | Africa Soil Grids | https://www.nature.com/articles/s41598-021-85639-y |  | AWY |  |
| Hydrologic Soil Group | Derived from soil physical properties, following USDA recommendations | 2020 | 250m | Raster | The Oak Ridge National Laboratory | https://daac.ornl.gov/cgi-bin/dsviewer.pl?ds_id=1566 |  | AWY modelling |  |
| Kc | Water use coefficient by land cover/land use type | 1992-2020 |  | .csv table | InVEST 9.1 user guide Kc calculator and FAO data | https://naturalcapitalproject.stanford.edu/sites/g/files/sbiybj9321/f/kc_calculator.xlsx |  | AWY modelling |  |
| Climate Zones | Rainfall statistics calculated at District level | 2001 |  | Raster |  | https://www.worldwildlife.org/publications/terrestrial-ecoregions-of-the-world |  | SWY modelling |  |
| Net Primary productivity (NPP) | MOD17A3HGF v006 MODIS/Terra Net Primary Production Gap-Filled Yearly L4 Global 500 m SIN Grid |  | 500 m | Raster | The United States Geological Survey (USGS) | https://lpdaac.usgs.gov/products/mod17a3hgfv006/ |  | NPP |  |
| Vegetation health Condition | Landsat 5,7 and 8 used for calculation of the Normalized vegetation Index (NDVI) | 1992-2020 | 30 m | Raster | The United States Geological Survey (USGS) |  |  | NDVI |  |
| Socio-economic factors |  |  |  |  |  |  |  |  |  |
| Livestock (cattle, sheep, and goat) | Livestock distribution in 2010 expressed in total number of specified animals per pixel. Gridded Livestock of the World database (GLW 3). The dasymetric method (DA) was selected | 2010 | 1 km m | Raster | FAO |  | https://www.fao.org/livestock-systems/global-distributions/cattle/en/ |  |  |
| Human Population grid | The Gridded Population of the World, Version 4 (GPWv4): Population Density Adjusted to Match 2015 Revision of UN WPP Country Totals | 2000,2005,2015,2020 | 1km | Raster | Center for International Earth Science Information Network - CIESIN - Columbia University. 2018 |  | https://sedac.ciesin.columbia.edu/data/set/gpw-v4-population-density-adjusted-to-2015-unwpp-country-totals-rev11/data-download |  |  |
| Protected areas boundary | From IUCN The World Database on Protected Areas (WDPA) |  |  |  |  |  | https://www.iucn.org/theme/protected-areas/our-work/world-database-protected-areas |  |  |

# InVEST model parameters

We derived sediment delivery ratio (SDR) model parameters of C and P factors primarily from 36,37. The cover factor C in cropland, we used values based on crop cover effectiveness in reducing soil erosion from Clay and Lewis (1990). We used P factors of 1 for all land cover types except for cropland which is 0.531. The P factor was an average based on field measurements reported in Rakotomamonjy et al. (2016) 38 from the Avaratrambolo watershed. A study by the Institut National des Sciences et Techniques Nucleaires (INSTN - Madagascar) team found that traditional terrace systems have an effect of reducing soil erosion by up 40% in eastern central highlands 39,40. We applied default values for Borselli IC0 and maximum SDR parameters and set threshold flow accumulation to 1,100 based on evaluation of the model-derived stream networks using various accumulation parameters 37.

Table 1: Biophysical table for Sediment delivery ratio

| LULC_desc | lucode | usle_c | usle_p |
| --- | --- | --- | --- |
| Tree Cover | 10 | 0.001 | 1 |
| Shrubland | 20 | 0.08 | 1 |
| Grassland | 30 | 0.08 | 1 |
| Cropland | 40 | 0.17 | 0.55 |
| Built-up | 50 | 0.1 | 1 |
| Bare Sparse vegetation | 60 | 0.45 | 1 |
| Water body | 80 | 0 | 1 |
| Wetland | 90 | 0.077 | 1 |
| Mangrove | 95 | 0.001 | 1 |

Table 2: Biophysical table for Sediment delivery ratio, with ILM implemented

| LULC_desc | lucode | usle_c | usle_p |
| --- | --- | --- | --- |
| Tree Cover | 10 | 0.001 | 1 |
| Shrubland + ILM | 20 | 0.08 | 0.85 |
| Grassland + ILM | 30 | 0.08 | 0.6 |
| Cropland + ILM | 40 | 0.17 | 0.4 |
| Built-up | 50 | 0.1 | 1 |
| Bare Sparse veg + ILM | 60 | 0.45 | 0.85 |
| Water body | 80 | 0 | 1 |
| Wetland | 90 | 0.077 | 1 |
| Mangrove | 95 | 0.001 | 1 |

Table 3: Biophysical table for Seasonal water yield model

| Description | lu code | Kc_1 | Kc_2 | Kc_3 | Kc_4 | Kc_5 | Kc_6 | Kc_7 | Kc_8 | Kc_9 | Kc_ 10 | Kc_ 11 | Kc_ 12 | CN_A | CN_B | CN_C | CN_D |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Tree Cover | 10 | 0.837 | 0.891 | 0.833 | 0.887 | 0.881 | 0.856 | 0.846 | 0.813 | 0.805 | 0.783 | 0.798 | 0.899 | 30 | 30 | 41 | 48 |
| Shrubland | 20 | 0.664 | 0.658 | 0.682 | 0.844 | 0.788 | 0.561 | 0.402 | 0.34 | 0.379 | 0.411 | 0.611 | 0.59 | 30 | 30 | 41 | 48 |
| Grassland | 30 | 0.688 | 0.676 | 0.711 | 0.835 | 0.845 | 0.704 | 0.585 | 0.488 | 0.503 | 0.545 | 0.617 | 0.67 | 30 | 30 | 41 | 48 |
| Cropland | 40 | 0.683 | 0.95 | 1.21 | 0.95 | 0.71 | 0.55 | 0.55 | 0.3 | 0.3 | 0.3 | 0.3 | 0.583 | 67 | 78 | 85 | 89 |
| Built-up | 50 | 0.2 | 0.475 | 0.475 | 0.45 | 0.45 | 0.2 | 0.2 | 0.25 | 0.25 | 0.25 | 0.25 | 0.2 | 91 | 91 | 91 | 91 |
| Bare/Sparse | 60 | 0.2 | 0.475 | 0.475 | 0.45 | 0.45 | 0.2 | 0.2 | 0.25 | 0.25 | 0.25 | 0.25 | 0.2 | 91 | 91 | 91 | 91 |
| Water body | 80 | 1.03 | 1.028 | 1.031 | 1.099 | 1.09 | 1.035 | 0.972 | 0.924 | 0.935 | 0.954 | 1.012 | 1.016 | 99 | 99 | 99 | 99 |
| Wetland | 90 | 0.73 | 0.722 | 0.733 | 0.886 | 0.858 | 0.733 | 0.579 | 0.469 | 0.509 | 0.549 | 0.701 | 0.697 | 49 | 69 | 79 | 84 |
| Mangrove | 95 | 1 | 1.2 | 1.2 | 1.2 | 1 | 1 | 0.5 | 0.5 | 0.5 | 0.5 | 0.5 | 1 | 49 | 69 | 79 | 84 |

Table 4: Biophysical table for Seasonal water yield model, with ILM implemented

| Description | lu code | Kc_1 | Kc_2 | Kc_3 | Kc_4 | Kc_5 | Kc_6 | Kc_7 | Kc_8 | Kc_9 | Kc_ 10 | Kc_ 11 | Kc_ 12 | CN_A | CN_B | CN_C | CN_D |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Tree Cover | 10 | 0.837 | 0.891 | 0.833 | 0.887 | 0.881 | 0.856 | 0.846 | 0.813 | 0.805 | 0.783 | 0.798 | 0.899 | 30 | 30 | 41 | 48 |
| Shrubland + ILM | 20 | 0.764 | 0.757 | 0.784 | 0.971 | 0.906 | 0.645 | 0.462 | 0.391 | 0.436 | 0.473 | 0.703 | 0.679 | 28.5 | 28.5 | 38.9 | 45.6 |
| Grassland + ILM | 30 | 0.826 | 0.811 | 0.853 | 1.002 | 1.014 | 0.845 | 0.702 | 0.586 | 0.604 | 0.654 | 0.740 | 0.804 | 27 | 27 | 36.9 | 43.2 |
| Cropland + ILM | 40 | 0.820 | 1.140 | 1.452 | 1.140 | 0.852 | 0.660 | 0.660 | 0.360 | 0.360 | 0.360 | 0.360 | 0.700 | 60.3 | 70.2 | 76.5 | 80.1 |
| Built-up | 50 | 0.2 | 0.475 | 0.475 | 0.45 | 0.45 | 0.2 | 0.2 | 0.25 | 0.25 | 0.25 | 0.25 | 0.2 | 91 | 91 | 91 | 91 |
| Bare/Sparse + ILM | 60 | 0.230 | 0.546 | 0.546 | 0.518 | 0.518 | 0.230 | 0.230 | 0.288 | 0.288 | 0.288 | 0.288 | 0.230 | 86.5 | 86.5 | 86.5 | 86.5 |
| Water body | 80 | 1.03 | 1.028 | 1.031 | 1.099 | 1.09 | 1.035 | 0.972 | 0.924 | 0.935 | 0.954 | 1.012 | 1.016 | 99 | 99 | 99 | 99 |
| Wetland | 90 | 0.73 | 0.722 | 0.733 | 0.886 | 0.858 | 0.733 | 0.579 | 0.469 | 0.509 | 0.549 | 0.701 | 0.697 | 49 | 69 | 79 | 84 |
| Mangrove | 95 | 1 | 1.2 | 1.2 | 1.2 | 1 | 1 | 0.5 | 0.5 | 0.5 | 0.5 | 0.5 | 1 | 49 | 69 | 79 | 84 |

Table 4: Climatic table for Seasonal water yield model

| ECO_NAME | cz_id | jan | feb | mar | apr | may | jun | jul | aug | sep | oct | nov | dec |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Madagascar dry deciduous forests | 2 | 26 | 24 | 21 | 6 | 3 | 4 | 3 | 1 | 1 | 10 | 14 | 22 |
| Madagascar lowland forests | 17 | 23 | 18 | 22 | 16 | 11 | 7 | 6 | 1 | 0 | 9 | 7 | 16 |
| Madagascar spiny thickets | 11 | 12 | 12 | 10 | 6 | 4 | 5 | 2 | 0 | 6 | 6 | 4 | 12 |
| Madagascar subhumid forests | 18 | 26 | 20 | 21 | 12 | 8 | 8 | 4 | 1 | 2 | 9 | 12 | 20 |
| Madagascar succulent woodlands | 12 | 18 | 16 | 8 | 5 | 5 | 4 | 1 | 0 | 4 | 3 | 7 | 14 |

# 7. References

1.	Biancalani, R., Nachtergaele, F., Petri, M. & Bunning, S. Land Degradation Assessment in Drylands: Methodology and Results. (2013).

2.	Kosmas, C. et al. Evaluation and Selection of Indicators for Land Degradation and Desertification Monitoring: Methodological Approach. Environ. Manage. 54, 951–970 (2014).

3.	Olsson, L. et al. Climate Change and Land. in IPCC special report on climate change, desertification, land degradation, sustainable land management, food security, and greenhouse gas fluxes in terrestrial ecosystems (2019).

4.	Allen, D. E., Singh, B. P. & Dalal, R. C. Soil Health Indicators Under Climate Change: A Review of Current Knowledge. 25–45 (2011) doi:10.1007/978-3-642-20256-8_2.

5.	Pettorelli, N. The normalized vegetation index. (Oxford University Press, 2013).

6.	Abdi, A. M., Seaquist, J., Tenenbaum, D. E., Eklundh, L. & Ardö, J. The supply and demand of net primary production in the Sahel. Environ. Res. Lett. 9, (2014).

7.	Robinson P. Nathaniel, Cindy M. Cox & Jawoo Koo. Harnessing net primary productivity data for monitoring sustainable development of agriculture. https://www.ifpri.org/publication/harnessing-net-primary-productivity-data-monitoring-sustainable-development-agriculture (2016).

8.	Sharp, E. R. et al. InVEST 3.5.0.post358+he23ea3e79185 User’s Guide. (2018).

9.	USGS. Base Flow in Rivers by Water Science School. https://www.usgs.gov/special-topics/water-science-school/science/base-flow-rivers (2018).

10.	Aladejana, O. O., Salami, A. T. & Adetoro, O. I. O. Hydrological responses to land degradation in the Northwest Benin Owena River Basin, Nigeria. J. Environ. Manage. 225, 300–312 (2018).

11.	USGS. MOD17A3HGF v006 MODIS/Terra Net Primary Production Gap-Filled Yearly L4 Global 500 m SIN Grid. https://lpdaac.usgs.gov/products/mod17a3hgfv006/ (2022).

12.	Styger, E., Rakotondramasy, H. M., Pfeffer, M. J., Fernandes, E. C. M. & Bates, D. M. Influence of slash-and-burn farming practices on fallow succession and land degradation in the rainforest region of Madagascar. Agric. Ecosyst. Environ. 119, 257–269 (2007).

13.	Wallace, C. S. A., Thenkabail, P., Rodriguez, J. R. & Brown, M. K. Fallow-land Algorithm based on Neighborhood and Temporal Anomalies (FANTA) to map planted versus fallowed croplands using MODIS data to assist in drought studies leading to water and food security assessments. GIScience Remote Sens. 54, 258–282 (2017).

14.	Zeng, L., Wardlow, B. D., Xiang, D., Hu, S. & Li, D. A review of vegetation phenological metrics extraction using time-series, multispectral satellite data. Remote Sens. Environ. 237, 111511 (2020).

15.	Steven, M. D., Malthus, T. J., Baret, F., Xu, H. & Chopping, M. J. Intercalibration of vegetation indices from different sensor systems. Remote Sens. Environ. 88, 412–422 (2003).

16.	Roy, D. P. et al. Characterization of Landsat-7 to Landsat-8 reflective wavelength and normalized difference vegetation index continuity. Remote Sens. Environ. 185, 57–70 (2016).

17.	Bullock, J. M. & Ding, H. A guide to selecting ecosystem service models for decision-making. (2018).

18.	Neugarten, R. A. et al. Tools for measuring, modelling, and valuing ecosystem services Guidance for Key Biodiversity Areas, natural World Heritage sites, and protected areas Best Practice Protected Area Guidelines Series No. 28. Best Practice Protected Area Guidelines Series (2018).

19.	Sharp et al. InVEST 3.9.0.post0+ug.gbbfa26d.d20201215 User’s Guide. National Capital Project http://data.naturalcapitalandresilienceplatform.org/invest-releases/documentation/current_release/index.html (2020).

20.	Scordo, F. et al. Modeling Water Yield: Assessing the role of site and region-specific attributes in determining model performance of the InVEST Seasonal Water Yield Model. Water (Switzerland) 10, 1–42 (2018).

21.	ESA Copernicus. Land cover classification gridded maps from 1992 to present derived from satellite observations. https://cds.climate.copernicus.eu/cdsapp#!/dataset/satellite-land-cover?tab=form (2021).

22.	Brockmann, C., Boettcher, M. & Kirches, G. Product Quality Assurance Document CDR Land Cover ( brokered from CCI Land Cover ). (European Space Agency (ESA), 2018).

23.	Tsendbazar, N. et al. World Cover Product Validation Report (D12-PVR). https://worldcover2020.esa.int/data/docs/WorldCover_PVR_V1.1.pdf (2021).

24.	FutureWater. Land Use Planning for Enhanced Resilience of Landscapes (LAUREL) Madagascar. https://www.futurewater.eu/projects/laurel/ (2020).

25.	UCSB. Index of /products/CHIRPS-2.0/africa_monthly/bils. https://data.chc.ucsb.edu/products/CHIRPS-2.0/africa_monthly/bils/ (2021).

26.	UCSB. Index of /products/Hobbins_RefET. https://data.chc.ucsb.edu/products/Hobbins_RefET/ (2021).

27.	Panagos, P. et al. Global rainfall erosivity assessment based on high-temporal resolution rainfall records. Sci. Rep. 7, 4175 (2017).

28.	Hengl, T. et al. African soil properties and nutrients mapped at 30 m spatial resolution using two-scale ensemble machine learning. Sci. Rep. 11, 1–18 (2021).

29.	Anache, J. A. A., Bacchi, C. G. V., Panachuki, E. & Sobrinho, T. A. Assessment of methods for predicting soil erodibility in soil loss modeling. Geociências 34, 32–40 (2015).

30.	Sharpley, A. N. & Williams, J. R. EPIC: The erosion-productivity impact calculator. Technical Bulletin http://agris.fao.org/agris-search/search.do?recordID=US9403696 (1990).

31.	Foster, G. R., McCool, D. K., Renard, K. G. & Moldenhauer, W. C. Conversion of the universal soil loss equation to SI metric units. J. Soil Water Conserv. 36, 355–359 (1981).

32.	Zhang, X., Wang, J., Gao, Y. & Wang, L. Variations and controlling factors of vegetation dynamics on the Qingzang Plateau of China over the recent 20 years. Geogr. Sustain. 2, 74–85 (2021).

33.	Wasserman, L. All of Statistics: A Concise Course in Statistical Inference. (Springer, 2004). doi:10.1007/978-0-387-21736-9_1.

34.	StatisticsHowTo. Linear Regression: Simple Steps, Video. Find Equation, Coefficient, Slope - Statistics How To. https://www.statisticshowto.com/probability-and-statistics/regression-analysis/find-a-linear-regression-equation/ (2022).

35.	Minitab. Understanding t-Tests: t-values and t-distributions. https://blog.minitab.com/en/adventures-in-statistics-2/understanding-t-tests-t-values-and-t-distributions (2016).

36.	Leh, M. D. K., Matlock, M. D., Cummings, E. C. & Nalley, L. L. Quantifying and mapping multiple ecosystem services change in West Africa. Agric. Ecosyst. Environ. 165, 6–18 (2013).

37.	Hamel, P. et al. Sediment delivery modeling in practice : Comparing the effects of watershed characteristics and data resolution across hydroclimatic regions. Sci. Total Environ. (2016) doi:10.1016/j.scitotenv.2016.12.103.

38.	Rakotomamonjy, E. T. Modelisation De L ’ Erosion Hydrique Des Sols Sur Un Bassin Versant d’Avaratrambolo. vol. 11 (Universite d’Antananarivo, 2016).

39.	Mabit, L. et al. Promoting the use of isotopic techniques to combat soil erosion: An overview of the key role played by the SWMCN Subprogramme of the Joint FAO/IAEA Division over the last 20 years. L. Degrad. Dev. 29, 3077–3091 (2018).

40.	Rabesiranana, N. et al. Assessment of soil redistribution rates by 137Cs and 210Pbex in a typical Malagasy agricultural field. J. Environ. Radioact. 152, 112–118 (2016).

Data were acquired from World Bank Madagascar team. Data are not published and not officially validated by the Government of Madagascar.

World Bank Group. 2020. State and trends of carbon pricing 2020

Forest Carbon Partenership Facility. 2018. ER Program Name and Country: Atiala Atsinanana Emission Reduction Program, Madagascar

Razafindralambo, R. Quoted on Hockey & Razafindralambo (2006) A Social Cost-Benefit Analysis of Conserving the Ranomafana-Andringitra-Pic d’Ivohibe Corridor; and van Kooten, G.C., Eagle, A.J., Manley, J., & Smolak, T. (2004) How costly are carbon offsets? A meta-analysis of carbon forest sinks. Environmental Science & Policy, 7, 239-251

Hockley, N. & Razafindralambo, R. 2006. A Social Cost-Benefit Analysis of Conserving the Ranomafana-Andringitra-Pic d’Ivohibe Corridor

Nordhaus, W. D. (2017). Revisiting the social cost of carbon. Proceedings of the National Academy of Sciences, 114(7), 1518-1523.

Adeogun, Niyi & Sule, B. & Salami, Adebayo. (2016). Cost effectiveness of sediment management strategies for mitigation of sedimentation at Jebba Hydropower reservoir, Nigeria. Journal of King Saud University - Engineering Sciences. 30. 10.1016/j.jksues.2016.01.003

The data source was the Office de Régulation de L'électricité Madagascar: 1) Inventaire du parc  hydroélectrique existant  Juin 2013, RI Antananarivo (RIA) ; 2)  Données Techniques, Statistiques de Ventes, Abonnés et Productions JIRAMA

Office de Régulation de L'électricité Madagascar : Donnés Techniques, Tarifs. (http://www.ore.mg/jargon.html).

Udayakumara, E.P., & Gunawardena, U. (2017). Reducing Siltation and Increasing Hydropower Generation from the Rantambe Reservoir, Sri Lanka

Brune, Gunnar M. 1953. Trap efficiency of reservoirs. Transactions, American Geophysical Union, 34(3), 407.

Annandale, G. W., Morris, G. L.Karki, P., 2016. Extending the life of reservoirs : sustainable sediment management for dams and run-of-river hydropower, World Bank Group. , Washington, D.C

WEDA’s Technical Report: “Reservoir Dredging: A Practical Overview”; WEDA, 2021 Available online at: Error! Hyperlink reference not valid.

Grimaldi, S., Angeluccetti, I., Coviello, V., & Vezza, P. (2013). Cost-Effectiveness of Soil and Water Conservation Measures on the Catchment Sediment Budget-The Laaba Watershed Case Study, Burkina Faso. Land Degradation & Development, 26(7), 737–747. doi:10.1002/ldr.2212

Both countries classified by the World Bank as low-income economies and with GDP (international dollars) of  46.08 billion $ in 2019 and   47.53 billion $ in 2020 in Burkina Faso and   42.76 billion $ in 2019 and 45.5 billion $ in 2020 Madagascar.

Lehner, B., C. Reidy Liermann, C. Revenga, C. Vörösmarty, B. Fekete, P. Crouzet, P. Döll, M. Endejan, K. Frenken, J. Magome, C. Nilsson, J.C. Robertson, R. Rodel, N. Sindorf, and D. Wisser. 2011. High-resolution mapping of the world’s reservoirs and dams for sustainable river-flow management. Frontiers in Ecology and the Environment 9 (9): 494-502.